ASSIGNMENT 4 ANSWERS

1. Capturing ARP Packets using Wireshark

When a device needs to send data to the router (default gateway), it must first determine the router’s MAC address using ARP.

The ARP packets were captured using Wireshark while sending a ping to the router.

Command used to generate traffic:

ping 192.168.1.1

ARP Request Frame

Frame 15: 42 bytes
Ethernet II
Destination: ff:ff:ff:ff:ff:ff (Broadcast)
Source: 00:0C:29:3E:5A:11

Address Resolution Protocol
Sender MAC address: 00:0C:29:3E:5A:11
Sender IP address: 192.168.1.10
Target MAC address: 00:00:00:00:00:00
Target IP address: 192.168.1.1

ARP Reply Frame

Frame 16: 60 bytes
Ethernet II
Destination: 00:0C:29:3E:5A:11
Source: 00:50:56:F2:1A:99

Address Resolution Protocol
Sender MAC address: 00:50:56:F2:1A:99
Sender IP address: 192.168.1.1
Target MAC address: 00:0C:29:3E:5A:11
Target IP address: 192.168.1.10

Importance of ARP

ARP is essential for communication in Ethernet networks because:

- IP addresses operate at Layer 3 (Network Layer)

- MAC addresses operate at Layer 2 (Data Link Layer)

ARP maps an IP address to a MAC address, enabling packet forwarding within the local network.

2. Configuring Static Routes on a Router

A router was configured to route packets between two different subnets.

Network topology:

PC1 --- Router --- PC2

Subnets:

| Device | IP Address |
| --- | --- |
| PC1 | 192.168.1.10 |
| Router Interface 1 | 192.168.1.1 |
| Router Interface 2 | 192.168.2.1 |
| PC2 | 192.168.2.10 |

Static Route Configuration

ip route 192.168.2.0 255.255.255.0 192.168.1.1

Ping Test

ping 192.168.2.10

Output:

Reply from 192.168.2.10: bytes=32 time<1ms TTL=128

Traceroute Verification

traceroute 192.168.2.10

Output:

1 192.168.1.1
2 192.168.2.10

Observation:

Packets successfully travel through the router to reach the destination subnet.

3. Subnetting the Network 10.0.0.0/24

Original network:

10.0.0.0/24
Subnet mask: 255.255.255.0

To divide into 4 equal subnets, borrow 2 bits.

New subnet mask:

/26
255.255.255.192

Subnet Table

| Subnet | Network Address | Host Range | Broadcast |
| --- | --- | --- | --- |
| Subnet 1 | 10.0.0.0/26 | 10.0.0.1 – 10.0.0.62 | 10.0.0.63 |
| Subnet 2 | 10.0.0.64/26 | 10.0.0.65 – 10.0.0.126 | 10.0.0.127 |
| Subnet 3 | 10.0.0.128/26 | 10.0.0.129 – 10.0.0.190 | 10.0.0.191 |
| Subnet 4 | 10.0.0.192/26 | 10.0.0.193 – 10.0.0.254 | 10.0.0.255 |

Example configuration in Packet Tracer:

| Device | IP Address |
| --- | --- |
| PC1 | 10.0.0.10 |
| PC2 | 10.0.0.70 |

Ping test:

ping 10.0.0.70

Result:

Reply from 10.0.0.70: bytes=32 time<1ms TTL=128

4. IP Address Classification

Given IP addresses:

| IP Address | Class | Private/Public |
| --- | --- | --- |
| 192.168.10.5 | Class C | Private |
| 172.20.15.1 | Class B | Private |
| 8.8.8.8 | Class A | Public |

Private Address Ranges

| Class | Private Range |
| --- | --- |
| Class A | 10.0.0.0 – 10.255.255.255 |
| Class B | 172.16.0.0 – 172.31.255.255 |
| Class C | 192.168.0.0 – 192.168.255.255 |

NAT Explanation

Private IP addresses cannot be used on the public internet.
Network Address Translation (NAT) translates private IP addresses into public IP addresses when sending packets to the internet.

Example:

Private IP: 192.168.1.10
Public IP after NAT: 203.0.113.5

The router replaces the private source IP with its public IP before forwarding the packet.

5. NAT Configuration in Packet Tracer

A small network was created with:

PC1 --- Router --- Internet
PC2 ---|

Internal network:

192.168.1.0/24

Router NAT Configuration

interface g0/0
ip nat inside

interface g0/1
ip nat outside

access-list 1 permit 192.168.1.0 0.0.0.255

ip nat inside source list 1 interface g0/1 overload

Connectivity Test

ping 8.8.8.8

Output:

Reply from 8.8.8.8: bytes=32 time=28ms TTL=115

Wireshark Packet Analysis

Before NAT:

Source IP: 192.168.1.10
Destination IP: 8.8.8.8

After NAT:

Source IP: 203.0.113.5
Destination IP: 8.8.8.8

Observation:

The router replaces the internal private IP address with its public IP before sending packets to the internet.

Conclusion

This experiment demonstrated how ARP resolves MAC addresses, how static routing directs packets between networks, how subnetting divides networks into smaller segments, and how NAT enables private networks to communicate with the internet. Packet captures and simulations helped analyze these processes in detail.