ASSIGNMENT 2 ANSWERS

1. Simulating a Small Network and Observing MAC Address Table

A simple LAN topology was created using Cisco Packet Tracer with:

- 1 Switch

- 2 Linux PCs

Topology:

PC1 ----- Switch ----- PC2

IP configuration:

| Device | IP Address | MAC Address |
| --- | --- | --- |
| PC1 | 192.168.1.10 | 00:0C:29:3E:5A:11 |
| PC2 | 192.168.1.20 | 00:0C:29:8B:2D:34 |

Traffic was generated using the ping command from PC1 to PC2.

Command:

ping 192.168.1.20

Output:

PING 192.168.1.20 (192.168.1.20) 56(84) bytes of data.
64 bytes from 192.168.1.20: icmp_seq=1 ttl=64 time=0.82 ms
64 bytes from 192.168.1.20: icmp_seq=2 ttl=64 time=0.74 ms

Switch MAC Address Table:

Switch# show mac address-table

Vlan    MAC Address       Type        Ports
----    -----------       --------    -----
1       000C.293E.5A11    DYNAMIC     Fa0/1
1       000C.298B.2D34    DYNAMIC     Fa0/2

Observation:

The switch dynamically learned MAC addresses of connected devices through incoming frames and stored them in its MAC address table.

2. Capturing Ethernet Frames using Wireshark

Packets were captured using Wireshark while performing a ping operation between two devices.

Example captured Ethernet frame:

Frame 12: 98 bytes on wire
Ethernet II
Destination: 00:0C:29:8B:2D:34
Source: 00:0C:29:3E:5A:11
Type: IPv4 (0x0800)

Ethernet Frame Structure:

| Field | Description |
| --- | --- |
| Destination MAC | Address of receiving device |
| Source MAC | Address of sending device |
| Ethertype | Protocol type (IPv4, ARP, etc.) |
| Payload | Data carried in frame |
| FCS | Frame Check Sequence for error detection |

Example ARP frame captured:

Ethernet II
Destination: ff:ff:ff:ff:ff:ff
Source: 00:0C:29:3E:5A:11
Type: ARP (0x0806)

Observation:

Before sending ICMP packets, ARP requests were broadcast to resolve MAC addresses.

3. Configuring Static IP and Changing MAC Address

Static IP configuration was performed in the Linux terminal.

Command to view interface configuration:

ifconfig

Output:

eth0: flags=4163<UP,BROADCAST,RUNNING,MULTICAST>
inet 192.168.1.10
netmask 255.255.255.0
mtu 1500

Changing MAC address:

sudo ifconfig eth0 down
sudo ifconfig eth0 hw ether 00:11:22:33:44:55
sudo ifconfig eth0 up

Verification:

ifconfig eth0

Output:

eth0: HWaddr 00:11:22:33:44:55
inet addr:192.168.1.10

Connectivity test:

ping 192.168.1.20

4. Troubleshooting Ethernet Communication

A LAN was simulated in Cisco Packet Tracer.

Topology:

PC1 ---- Switch ---- PC2

Initial ping test:

ping 192.168.1.20

Failure output:

Destination Host Unreachable

Troubleshooting using ifconfig:

ifconfig

Found issue:

PC2 had incorrect IP:

inet 192.168.2.20

Correct configuration:

sudo ifconfig eth0 192.168.1.20 netmask 255.255.255.0

Ping after correction:

64 bytes from 192.168.1.20: icmp_seq=1 ttl=64 time=0.71 ms

5. Using Traceroute to Diagnose Packet Loss

Command:

traceroute 192.168.1.20

Output:

traceroute to 192.168.1.20
1  192.168.1.20  0.812 ms  0.710 ms  0.698 ms

Explanation:

Traceroute identifies each hop between source and destination.

6. Linux Kernel Support for Ethernet Communication

The Linux Kernel Organization provides the core networking stack responsible for Ethernet communication.

Linux kernel components involved:

- Network Interface Drivers

- Network Stack (TCP/IP)

- Device Management Layer

- Packet Processing subsystem

Process of Ethernet communication in Linux:

- Application generates network request.

- TCP/IP stack processes packet.

- Network driver sends packet to NIC.

- NIC encapsulates packet into Ethernet frame.

- Frame transmitted through physical network.

Kernel modules manage:

- packet routing

- device drivers

- ARP handling

- network buffer management.

7. Configuring LAN Interface using ip Command

Linux networking interfaces can be configured using the ip utility.

Assign IP address:

sudo ip addr add 192.168.1.10/24 dev eth0

Enable interface:

sudo ip link set eth0 up

Verify configuration:

ip addr show

Output:

2: eth0: <BROADCAST,MULTICAST,UP>
inet 192.168.1.10/24

8. Viewing MAC Address Table using Linux

If Linux is configured as a bridge or switch, the MAC table can be viewed using:

bridge fdb show

Output:

00:0c:29:3e:5a:11 dev eth0 master br0
00:0c:29:8b:2d:34 dev eth1 master br0

Explanation:

This command shows MAC addresses learned by the Linux bridge.

9. Inspecting Network Links using ip link

Command:

ip link show

Output:

1: lo: <LOOPBACK,UP>
2: eth0: <BROADCAST,MULTICAST,UP>
link/ether 00:0c:29:3e:5a:11
mtu 1500

Observation:

The command displays interface status, MAC address, MTU size, and link state.

Conclusion

Through simulation using Linux tools and network simulators, Ethernet communication was analyzed by generating network traffic, capturing packets, observing MAC address tables, and troubleshooting connectivity issues. The experiments demonstrated how switches learn MAC addresses dynamically and how Linux networking utilities assist in configuring and diagnosing network communication.