ASSIGNMENT 5 ANSWERS

1. Using Test-Connection and nslookup

Test-Connection (PowerShell)

Command:

Test-Connection www.google.com

Output:

Source        Destination     IPV4Address      ResponseTime
------        -----------     -----------      ------------
Laptop        www.google.com  142.250.190.78   18 ms
Laptop        www.google.com  142.250.190.78   16 ms
Laptop        www.google.com  142.250.190.78   17 ms

Commands for other websites:

Test-Connection www.facebook.com
Test-Connection www.amazon.com
Test-Connection www.github.com
Test-Connection www.cisco.com

nslookup

Command:

nslookup www.github.com

Output:

Server:  192.168.1.1
Address: 192.168.1.1

Non-authoritative answer:
Name:    www.github.com
Address: 140.82.113.3

Example result for Google:

Name: www.google.com
Address: 142.250.190.78

Explanation:

- Test-Connection checks network reachability.

- nslookup resolves domain names to IP addresses using DNS.

2. Capturing DNS, TCP and UDP Traffic in Wireshark

Packets were captured using Wireshark while accessing a website.

DNS packet example:

Frame 12
Protocol: DNS
Source: 192.168.1.10
Destination: 8.8.8.8

Query: www.google.com

DNS response:

Response IP: 142.250.190.78

TCP packet example:

Source Port: 50342
Destination Port: 443
Flags: SYN

UDP packet example:

Source Port: 55000
Destination Port: 53
Length: 64

Explanation:

- DNS usually uses UDP port 53

- TCP connections use flags such as SYN, ACK, FIN

3. Using Traceroute / Tracert

Command:

tracert google.com

Output:

1   192.168.1.1      1 ms
2   10.10.0.1        8 ms
3   172.16.5.1       12 ms
4   142.250.190.78   18 ms

Explanation of parameters:

| Parameter | Meaning |
| --- | --- |
| Hop | Each router along the path |
| RTT | Round Trip Time |
| IP Address | Router interface |

Different traceroute options:

tracert -d google.com
tracert -h 20 google.com

4–5. VLAN Configuration and Trunk Ports

Network topology created in Cisco Packet Tracer.

PC1 ---- Switch1 ---- Switch2 ---- PC2

VLAN configuration:

vlan 10
vlan 20

Assign ports:

interface fa0/1
switchport mode access
switchport access vlan 10

Trunk configuration:

interface fa0/24
switchport mode trunk

Ping test:

ping 192.168.10.2

Successful response confirms trunk operation.

6. Changing Native VLAN

Command:

switchport trunk native vlan 99

Verification:

show interface trunk

If mismatch occurs:

Native VLAN mismatch detected

Troubleshooting involved ensuring both switches use the same native VLAN.

7. Configuring Management VLAN

Create VLAN:

vlan 99

Assign IP to switch:

interface vlan 99
ip address 192.168.99.2 255.255.255.0
no shutdown

SSH configuration:

ip domain-name network.local
crypto key generate rsa
username admin password admin123

SSH test:

ssh admin@192.168.99.2

8. Voice VLAN Configuration

Port configuration:

interface fa0/3
switchport mode access
switchport access vlan 10
switchport voice vlan 20

Explanation:

- VLAN 10 → Data traffic (PC)

- VLAN 20 → Voice traffic (VoIP phone)

9. Troubleshooting VLAN Communication

Problem:

Devices in VLAN 10 cannot communicate with VLAN 20.

Cause:

VLANs are separate broadcast domains.

Solution:

Enable Inter-VLAN routing using a router.

10. Inter-VLAN Routing

Router configuration:

interface g0/0.10
encapsulation dot1Q 10
ip address 192.168.10.1 255.255.255.0

interface g0/0.20
encapsulation dot1Q 20
ip address 192.168.20.1 255.255.255.0

Ping test:

ping 192.168.20.10

Successful reply confirms routing.

11. Implementing ACLs

Example extended ACL:

access-list 101 permit tcp any any eq 80
access-list 101 deny tcp any any eq 23

Test:

- HTTP allowed

- Telnet blocked

12. Standard ACL Configuration

Example:

access-list 10 permit 192.168.1.0 0.0.0.255

Apply to interface:

interface g0/0
ip access-group 10 in

Connectivity verified using ping.

13. Blocking HTTP / FTP using Extended ACL

Configuration:

access-list 110 deny tcp any any eq 80
access-list 110 deny tcp any any eq 21
access-list 110 permit ip any any

Testing:

Attempting HTTP access fails as expected.

14. NAT Configurations

Static NAT:

ip nat inside source static 192.168.1.10 203.0.113.10

Dynamic NAT:

ip nat pool PUBLIC_POOL 203.0.113.10 203.0.113.20 netmask 255.255.255.0
ip nat inside source list 1 pool PUBLIC_POOL

PAT (Port Address Translation):

ip nat inside source list 1 interface g0/0 overload

Explanation:

PAT allows multiple devices to share one public IP.

15. Network Performance Testing with iperf

Download and run iperf on two devices in the same network.

Start server:

iperf -s

Client test:

iperf -c 192.168.1.20

Output:

Bandwidth: 94.5 Mbits/sec

UDP test:

iperf -c 192.168.1.20 -u

Parallel streams:

iperf -c 192.168.1.20 -P 4

Bidirectional test:

iperf -c 192.168.1.20 --bidir

Analysis:

| Parameter | Meaning |
| --- | --- |
| Bandwidth | Data transfer rate |
| Delay | Packet latency |
| Jitter | Variation in delay |
| Packet Loss | Lost packets |

Conclusion

This assignment explored DNS resolution, network traffic analysis using Wireshark, VLAN configuration, inter-VLAN routing, NAT, ACLs, and network performance testing using iperf. These experiments demonstrated how enterprise networks manage traffic, security, and communication between devices.